CS19541 – COMPUTER NETWORKS LAB

PRACTICAL – 8

# AIM

To discover live hosts using Nmap scans (ARP, ICMP, TCP/UDP) on the TryHackMe Nmap room.

# REQUIREMENTS

Nmap and a suitable lab/authorized target network. The practical source specifies the TryHackMe Nmap room as the platform.

# NMAP COMMANDS / CODE

1. Discover live hosts without port scanning
nmap -sn TARGETS

2. ARP host discovery on the local subnet
nmap -PR -sn TARGETS

3. ICMP Echo discovery
nmap -PE -sn TARGETS

4. ICMP Timestamp discovery
nmap -PP -sn TARGETS

5. ICMP Address Mask discovery
nmap -PM -sn TARGETS

6. TCP SYN Ping
nmap -PS80 TARGETS

7. TCP SYN Ping on Telnet port
nmap -PS23 TARGETS

8. TCP ACK Ping
nmap -PA80 TARGETS

9. UDP host discovery
nmap -PU TARGETS

10. Disable reverse DNS lookup
nmap -n -sn TARGETS

11. Reverse-DNS lookup for all possible hosts
nmap -R TARGETS

12. Scan targets listed in a file
nmap -iL list_of_hosts.txt

13. List targets without scanning them
nmap -sL TARGETS

# INPUT / TARGET

Example target: 10.10.12.8
Example command: nmap -sn 10.10.12.8

# SAMPLE OUTPUT

Starting Nmap
Nmap scan report for 10.10.12.8
Host is up.
Nmap done: 1 IP address (1 host up) scanned.

# IMPORTANT PRACTICAL OBSERVATIONS

• ARP requests discover live hosts on the same subnet because ARP is a link-layer protocol.

• ICMP Echo uses -PE; Timestamp uses -PP; Address Mask uses -PM.

• TCP SYN Ping uses -PS and TCP ACK Ping uses -PA.

• The source specifies TCP SYN Ping as not requiring a privileged account and TCP ACK Ping as requiring a privileged account.

• For TCP SYN Ping on Telnet port 23, use -PS23.

• For UDP discovery, a closed UDP port may produce an ICMP 'port unreachable' response, indicating that the host is active.

• The -n option prevents reverse-DNS resolution, while -R requests reverse-DNS lookup for all possible hosts.

# RESULT

Thus, live hosts were discovered using Nmap host-discovery techniques based on ARP, ICMP, TCP and UDP, and the required Nmap commands were executed/verified in the authorized lab environment.